ENVIRONICS SERIES 4000/4040 – QUICK START INSTRUCTIONS

Windows 7/8/10

The enclosed Environics USB drive contains the S4000 software, instrument data specific to this system and the S4000 user manual in PDF format.

SOFTWARE INSTALLATION (version 2.x.x)

The software installation process installs the main application program and required support files.  The software needs to be installed only once, regardless of the number of instruments purchased.

- Close all running Windows applications

- Insert the Environics USB drive into a USB port

- If AutoPlay is enabled, select "Open folder to view files".  Otherwise, click Start – Computer and select the USB drive labelled ENVIRONICS

- Double-click the EnvironicsMenu icon

- Select "Install Environics Series 4000 software V2 (Windows 7)".  Follow the on-screen instructions.

USB DRIVER INSTALLATION

If connecting your system to a USB port, the correct USB driver must be installed.  
No driver is required for connection to a PC serial port.

For Environics systems with built-in USB port

- Select "Install S4000 built-in USB port driver (Win7)" and follow the on-screen instructions

For external USB-Serial adapter

- Select "Install Sabrent USB-Serial adapter driver (Win7)" and follow the on-screen instructions.

INSTRUMENT DATA INSTALLATION

The instrument data contains detailed calibration and configuration data that is unique to each instrument.

The instrument data for each physical instrument must be installed before it can be operated.

- Start the Environics software (Start – Environics – Environics)

- Click Instruments.  Click Import

- In the "Open" window, locate the USB drive.  Select the DATA folder, then the Serial # folder.  Select the file XXXXDATA.TXT, then click OK.

NOTE:  If your Mass Flow Controllers are calibrated in more than one gas, the DATA folder will contain additional folders for each calibration gas.  Repeat the process above, selecting each gas folder.

USB/COM PORT CONFIGURATION

The software communicates with the instrument via a USB or Serial COM port.  To select the serial port:

- Start the Environics software (Start – Environics – Environics)

- Click Instruments.  Click Select Serial Port.

- Select the desired serial port

USER MANUAL

The Series 4000 user manual is located on the Environics USB drive in PDF format.

From the Environics Menu, click the USER MANUALS button, then select Series 4000.

Alternatively, you can click Start – Windows Explorer, and browse to the USB drive – "manuals" folder

You can copy the user manual PDF file to your local drive for easy access

IMPORTANT: If the unit was accompanied by a system operator addendum, please read carefully before running gases through the unit. There will specific details to the setup and operation of the unit based on its unique configuration.